Pre-Digester EA Degradation Model

Reaction Water Block Calculation Chain

Technical Reference for Digester EA Charge Calculation

# 1. Purpose

The white liquor produced by the recausticizing plant has an Effective Alkali (EA) concentration of 86 g Na₂O/L. However, before the liquor enters the digester, the cooking chemicals react with wood components, consuming OH⁻ and generating reaction water and dissolved organics. This increases the total liquor volume while reducing the active chemical mass, resulting in a lower effective EA concentration at the digester.

This document provides the complete calculation chain to convert the recaust EA into the true digester EA, replicating the WinGEMS reaction water block logic. The agent must use the output of this model (not the raw recaust EA) for digester loading calculations.

# 2. Chemical Basis

## 2.1 Neutralization Reactions (OH to Water)

During cooking, NaOH reacts with acidic wood components (acetic acid, formic acid, uronic acids from hemicellulose dissolution):

NaOH + R-COOH → R-COONa + H₂O

Each mole of OH⁻ (MW = 17 g/mol) consumed produces one mole of H₂O (MW = 18 g/mol). The conversion factor 17/18 is the stoichiometric mass ratio: for every 18 g of water produced, 17 g of OH⁻ were consumed. This converts lb of reaction water produced into lb of OH⁻ consumed.

## 2.2 Organic Dissolution Reactions (OH to Organics)

OH⁻ is also consumed by delignification and carbohydrate degradation (peeling) reactions, where lignin and hemicellulose fragments dissolve into the liquor phase:

Lignin-OCH₃ + NaOH → Lignin-O⁻ Na⁺ + CH₃OH

Carbohydrate end-groups + NaOH → Hydroxy acids (sodium salts)

These dissolved organics add mass and volume to the liquor stream.

# 3. Input Parameters

| Parameter | Value | Unit | Description |
| --- | --- | --- | --- |
| Reaction Water | 48.6 | lb H₂O / BDT wood | Water produced per BDT of wood from neutralization reactions |
| OH to Organics | 64 | lb OH / BDT wood | OH consumed by organic dissolution per BDT of wood |
| Wood Flow | 83.33 | BD mt/hr | Bone-dry wood feed rate to digester |
| MT to Short Ton | 1.10 | short ton / mt | Metric ton to short ton conversion |
| WL EA Concentration | 0.0438587 | lb OH / lb liquor | EA in white liquor on OH basis (before reaction water) |
| WL EA after Rxn Water | 0.0289046 | lb OH / lb liquor | Depleted EA after reaction water step (used for organics calc) |
| Wood Flow (initial) | 83.33 | BD mt/hr | Bone-dry wood feed rate (used for water calc) |
| Wood Flow (after rxn water) | 83.0221 | BD mt/hr | Adjusted wood flow after initial dissolution (used for organics calc) |
| WL Mass Flow | 126.708 | mt/hr | White liquor total mass flow to digester |
| lb per mt | 2204.6 | lb / mt | Metric ton to pounds conversion |
| MW of OH⁻ | 17 | g/mol | Molecular weight of hydroxide ion (numerator in 17/18) |
| MW of H₂O | 18 | g/mol | Molecular weight of water (denominator in 17/18) |

# 4. Calculation Chain

## 4.1 Formula 1: % OH to Water in REACT

This calculates the percentage of total OH⁻ entering the digester that is consumed by neutralization reactions producing water.

Formula:

% OH to Water = [Reaction Water (lb H₂O/BDT) × Wood Flow (BD mt/hr)

× 1.10 (short ton/mt) × (17/18)]

/ [EA conc (lb OH/lb liq) × WL Flow (mt/hr)

× 2204.6 (lb/mt)] × 100

Numerator breakdown:

The numerator converts reaction water produced (lb H₂O/BDT) into lb OH consumed per hour:

48.6 × 83.33 × 1.10 = 4,452.2 lb H₂O/hr

4,452.2 × (17/18) = 4,204.8 lb OH consumed/hr

Why 17/18: The 48.6 value is in lb of water produced. Since 1 mol OH⁻ (17 g) produces 1 mol H₂O (18 g), the mass ratio 17/18 converts lb water produced back to lb OH⁻ consumed. This is a direct stoichiometric conversion.

Denominator breakdown:

The denominator calculates the total OH entering the digester in the white liquor:

0.0438587 × 126.708 × 2,204.6 = 12,253.0 lb OH/hr

Result:

% OH to Water = (4,204.8 / 12,253.0) × 100 = 34.32%

WinGEMS block value: 34.096% (difference due to rounding in displayed input values).

## 4.2 Formula 2: % OH to Organics in REACT

This calculates the percentage of OH⁻ consumed by organic dissolution reactions. CRITICAL: This calculation is performed AFTER the reaction water step, using the depleted EA concentration and adjusted wood flow.

Formula:

% OH to Organics = [OH to Organics (lb OH/BDT) × Wood Flow after Rxn (BD mt/hr)

× 1.10 (short ton/mt)]

/ [EA after Rxn Water (lb OH/lb liq) × WL Flow (mt/hr)

× 2204.6 (lb/mt)] × 100

Key difference from Formula 1: The organics calculation uses (a) the wood flow AFTER the reaction water step (83.0221 BD mt/hr, slightly reduced due to initial dissolution), and (b) the DEPLETED EA concentration (0.0289046 lb OH/lb liquor) after reaction water has already consumed its share of OH⁻. No 17/18 factor is needed because the 64 lb/BDT is already in lb OH, not lb water.

Numerator:

64 × 83.0221 × 1.10 = 5,842.8 lb OH/hr

Denominator (using depleted EA after reaction water):

0.0289046 × 126.708 × 2,204.6 = 8,075.6 lb OH/hr

Result:

% OH to Organics = (5,842.8 / 8,075.6) × 100 = 72.35%

WinGEMS block value: 72.3878% (confirmed match).

Sequential Logic Confirmed: The reaction water block operates in two sequential steps. Step 1 consumes OH for water reactions and depletes the EA from 0.0438587 to 0.0289046 lb OH/lb liquor. Step 2 then consumes OH for organics using this depleted concentration as its basis. This is why the % OH to Organics appears high (72.4%) — it is relative to the remaining OH, not the original total.

## 4.3 Formula 3: Residual EA to Digester

The block operates sequentially. After both reaction steps, we can calculate the residual EA.

Step 1 — Total OH entering in WL (lb/hr):

OH_total = 0.0438587 × 126.708 × 2,204.6 = 12,253 lb OH/hr

Step 2 — OH consumed by reaction water (lb/hr):

OH_water = 48.6 × 83.33 × 1.10 × (17/18) = 4,205 lb/hr

Step 3 — OH remaining after reaction water:

OH_after_water = 12,253 - 4,205 = 8,048 lb OH/hr

EA_depleted = 0.0289046 lb OH/lb liquor (as confirmed by WinGEMS)

Step 4 — OH consumed by organics (lb/hr):

OH_organics = 64.0 × 83.0221 × 1.10 = 5,843 lb/hr

Step 5 — Residual OH after both reactions (lb/hr):

OH_residual = 8,048 - 5,843 = 2,205 lb OH/hr

Step 4 — Volume increase:

The total liquor volume increases due to: (a) reaction water generated, and (b) dissolved organics entering the liquid phase. The new total volume determines the effective concentration:

V_WL = WL mass flow / WL density

V_reaction_water = Reaction water mass / water density

V_dissolved_organics = f(dissolved solids entering liquor)

V_total = V_WL + ΔV_water + ΔV_organics

Step 5 — Effective EA concentration:

EA_effective (g Na₂O/L) = [OH_residual (lb/hr) × (62/34)] / V_total

Where 62/34 converts from OH basis to Na₂O basis (MW Na₂O = 62, 2 × MW OH = 34).

This EA_effective is the value the agent must use for digester charge calculations, not the 86 g Na₂O/L from recaust.

# 5. Summary Table

| Calculation Step | Value |
| --- | --- |
| Total OH in WL (lb/hr) | 12,253 |
| OH consumed by water reactions (lb/hr) | 4,205 |
| OH remaining after water reactions (lb/hr) | 8,048 |
| EA after reaction water (lb OH/lb liq) | 0.0289046 |
| OH consumed by organic reactions (lb/hr) | 5,843 |
| Total OH consumed (lb/hr) | 10,048 |
| Residual OH to digester (lb/hr) | 2,205 |
| % OH consumed (total) | 82.1% |
| Recaust EA (raw) | 86 g Na₂O/L |
| Effective EA to digester | Must be recalculated with volume correction |

# 6. Important Notes for the Agent

1. Never use the recaust EA (86 g Na₂O/L) directly as the digester EA charge. Always pass the white liquor through this reaction water calculation first.

2. The 17/18 conversion factor applies ONLY to the reaction water parameter (48.6) because it is expressed in lb H₂O produced, not lb OH consumed. It converts water mass to the OH mass that produced it (stoichiometric ratio: MW OH / MW H₂O = 17/18).

3. The OH to Organics parameter (64) is already in lb OH/BDT and does NOT require the 17/18 factor.

4. The reaction water block operates SEQUENTIALLY: first consume OH for water reactions (depleting the EA concentration), then consume OH for organics using the depleted EA as the basis. This is why % OH to Organics = 72.4% — it is relative to the remaining OH after water reactions, not the original total.

5. The wood flow also changes slightly between steps (83.33 → 83.0221 BD mt/hr) due to initial dissolution in the reaction water step.

6. The EA charge should ultimately be expressed as % on OD wood for digester control, calculated as: EA (% on OD wood) = (OH_residual × 62/34) / (Wood flow in lb/hr) × 100.

7. The volumetric flow to the digester will be HIGHER than the WL feed flow because of the added reaction water and dissolved organics. This is expected and must be accounted for in hydraulic calculations.

1. How CE is used — the "hardwired" baseline

There are TWO places CE appears:

a) In the slaker (slaker_model.py:145): Uses the actual CE input to compute real WL composition:

conversion_fraction = causticity / (2 - causticity)   # e.g., 81% → 0.681

na2co3_conv_na2o = causticity * (gl_tta_g_L - gl_na2s_g_L) - gl_naoh_g_L

This correctly computes how much Na₂CO₃ the slaker converts to NaOH. Higher CE → more conversion → more NaOH in WL.

b) In the NaOH EA demand sizing (orchestrator.py:327): Uses a hardcoded BASELINE_CE = 0.81:

ea_at_baseline_ce_g_L = BASELINE_CE * non_sulfide_alkali_g_L + 0.5 * slaker_na2s_g_L

ea_at_actual_ce_g_L   = causticity  * non_sulfide_alkali_g_L + 0.5 * slaker_na2s_g_L

ea_deficit_g_L = max(0.0, ea_at_baseline - ea_at_actual)

This only fires when CE < 81% (EA deficit > 0). At CE ≥ 81%, the max(0,...) clamp makes it zero, so the EA constraint never activates. My plan reuses this BASELINE_CE = 0.81 as the reference for the flow-ratio calculation.

2. How the slaker computes NaOH conversion

From slaker_model.py lines 124–275, the full first-principles causticizing:

Reaction: Na₂CO₃ + Ca(OH)₂ → 2NaOH + CaCO₃

Step 1: GL composition (from dissolving tank)

GL_NaOH  = GL_AA - GL_Na₂S           (line 121)

GL_Na₂CO₃ = GL_TTA - GL_AA           (line 122) — this is the "dead load"

Step 2: Conversion fraction from target CE

x = CE / (2 - CE)                     (line 145)

CE 75%: x = 0.600, CE 81%: x = 0.681, CE 85%: x = 0.739

Step 3: Amount converted (g Na₂O/L)

na2co3_conv = CE × (TTA - Na₂S) - existing_NaOH    (line 151)

Step 4: WL species mass balance (lines 262-275)

WL_Na₂S   = GL_Na₂S  (inert — does NOT react)

WL_Na₂CO₃ = GL_Na₂CO₃ - converted amount

WL_NaOH   = GL_NaOH + converted amount + lime soda (0.17% Na₂O in lime)

WL_TTA    = Na₂S + NaOH + Na₂CO₃  (conserved — just redistributed)

So the slaker correctly produces more NaOH at higher CE (and less Na₂CO₃). The WL EA goes up. But the WL TTA stays the same — CE only redistributes internal Na, it doesn't create or destroy any.

That's why the Na losses approach gives constant NaOH — the total Na in the system doesn't change with CE. The flow-dependent washable soda adjustment captures the secondary effect: more WL flow at lower CE → more Na washed out → slightly

higher losses → slightly more NaOH needed.

Shall I proceed with the plan as written, or would you like any adjustments?

✻ Sautéed for 24m 13s

❯ another question before proceeed the ce should not be calculated instad of being and input , the ce would be calculate ful first principles Na2

CO3

+Ca(OH)2

⇌2NaOH+CaCO3

(s) and use a standard lime charge mole of 0.855 moles can + ca(oh)2 of lime / moles co3. in green liquor what would be inplications if we do it in that way? or shoudl we keep it hard wire?

⏺ Great question — this gets at causality. Let me analyze both approaches.

How CE Works Currently (Input)

User sets CE = 81%  →  conversion_fraction = 0.81/(2-0.81) = 0.681

→  Lime required = stoich_for_ALL_Na₂CO₃ × 0.85 (charge ratio)

→  WL NaOH = CE × (TTA - Na₂S)

CE drives lime. But in real mills, lime drives CE — the mill controls lime feed rate, and CE is what you GET.

What "Calculate CE" Would Mean

Reverse the causality:

User sets lime_charge_ratio = 0.855  →  moles_CaO_available = lime × CaO_purity / 56.08

→  moles_Na₂CO₃ = GL_Na₂CO₃ / 106

→  conversion x = f(mole_ratio, efficiency)

→  CE = 2x / (1+x)  ← OUTPUT

The Problem: Equilibrium vs. Stoichiometry

Pure stoichiometry at lime charge = 0.855:

- If ALL fed CaO reacts: x = 0.855, CE = 2×0.855/(1+0.855) = 92.2%

- But real mills get CE ~80-82% at the same charge

The gap exists because:

1. Equilibrium limitation — the reaction doesn't go to 100% completion

2. Dead-burned lime — some CaO is unreactive

3. Kinetic limitation — residence time matters

4. Temperature/concentration effects

To get CE=81% from a charge ratio of 0.855, you'd need a causticizing efficiency factor (~0.80) representing what fraction of the available CaO actually participates:

effective_mole_ratio = lime_charge × causticizing_efficiency

= 0.855 × 0.80 = 0.684

CE = 2×0.684 / (1+0.684) = 81.3%  ← matches reality

So you'd be trading one calibration parameter (CE) for two (lime_charge + causticizing_efficiency). The physics is more correct, but you haven't eliminated the need for a tuning knob.

Implications for NaOH Sensitivity

Key insight: Computing CE from lime charge does NOT automatically fix the NaOH sensitivity. Here's why:

- If lime charge is fixed at 0.855, CE is constant (~81%) — NaOH doesn't change

- If the user varies lime charge, CE varies — but NaOH is still from fixed Na losses (same problem)

The flow-dependent washable soda adjustment is needed regardless of whether CE is input or calculated. The mechanism (CE → WL demand → washable soda → NaOH) is the same either way.

Recommendation: Keep CE as Input

┌──────────────────┬──────────────────────────────────────┬──────────────────────────────────────┐

│      Factor      │             CE as Input              │            CE Calculated             │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ What-if ease     │ "What if CE drops to 75%?" — trivial │ Need to reverse-engineer lime charge │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ Calibration      │ Matches plant CE exactly             │ Needs efficiency factor to match     │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ Causality        │ Backwards (CE → lime)                │ Correct (lime → CE)                  │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ Complexity       │ Simple                               │ Adds equilibrium model               │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ NaOH fix needed? │ Yes (flow-dependent)                 │ Still yes                            │

├──────────────────┼──────────────────────────────────────┼──────────────────────────────────────┤

│ Parameters       │ 1 (CE)                               │ 2 (lime charge + efficiency)         │

└──────────────────┴──────────────────────────────────────┴──────────────────────────────────────┘

For what-if scenarios, CE as input is far more practical. Operators think "what if our CE drops?" not "what if our lime charge drops by 0.03 moles?". And the model already correctly computes WL composition from whatever CE you give it.

If you want to add calculated CE later, it could be a separate mode: "Auto CE from lime charge" toggle. But it's a separate enhancement from the NaOH sensitivity fix.